import pandas as pd

import numpy as np

import matplotlib.pyplot as plt

from sklearn.ensemble import RandomForestClassifier

from sklearn.tree import DecisionTreeClassifier

from sklearn.neighbors import KNeighborsClassifier

from sklearn.metrics import accuracy_score

from sklearn.model_selection import train_test_split

# --- 1. SENSOR MAPPING (NASA C-MAPSS OFFICIAL LABELS) ---

# Mapping generic IDs to physical descriptions and numbers for the presentation

sensor_labels = {

's1': 'S1: Fan Inlet Temp (R)',

's2': 'S2: LPC Outlet Temp (R)',

's3': 'S3: HPC Outlet Temp (R)',

's4': 'S4: LPT Outlet Temp (R)',

's5': 'S5: Fan Inlet Press (psia)',

's6': 'S6: Bypass Duct Press (psia)',

's7': 'S7: Total HPC Outlet Press (psia)',

's8': 'S8: Physical Fan Speed (rpm)',

's9': 'S9: Physical Core Speed (rpm)',

's10': 'S10: Engine Pressure Ratio',

's11': 'S11: HPC Outlet Static Press (psia)',

's12': 'S12: Ratio of Fuel Flow to Ps30',

's13': 'S13: Corrected Fan Speed (rpm)',

's14': 'S14: Corrected Core Speed (rpm)',

's15': 'S15: Bypass Ratio',

's16': 'S16: Burner Fuel-Air Ratio',

's17': 'S17: Bleed Enthalpy',

's18': 'S18: Demanded Fan Speed',

's19': 'S19: Demanded Corrected Fan Speed',

's20': 'S20: HPT Coolant Bleed (lbm/s)',

's21': 'S21: LPT Coolant Bleed (lbm/s)'

}

# --- 2. DATA LOADING ---

col_names = ['id', 'cycle', 'setting1', 'setting2', 'setting3'] + [f's{i}' for i in range(1, 22)]

try:

# Analyzing the train_FD001.txt file

df = pd.read_csv('train_FD001.txt', sep='\s+', header=None, names=col_names)

print(f"Successfully loaded {len(df)} rows of data.")

except FileNotFoundError:

print("Error: train_FD001.txt not found in the current directory.")

df = pd.DataFrame()

if not df.empty:

# --- 3. INTELLIGENT PRE-PROCESSING ---

sensors = [f's{i}' for i in range(1, 22)]

# Labeling: 1 if failure is imminent (within 30 cycles), 0 otherwise

df['max_cycle'] = df.groupby('id')['cycle'].transform('max')

df['label'] = (df['max_cycle'] - df['cycle'] <= 30).astype(int)

# Features: Using the 21 sensors

X = df[sensors]

y = df['label']

# Train-Test Split (The "Blind Test")

X_train, X_test, y_train, y_test = train_test_split(X, y, test_size=0.2, random_state=42)

# --- 4. MODEL TRAINING (Proposed Random Forest) ---

model_rf = RandomForestClassifier(n_estimators=100, random_state=42).fit(X_train, y_train)

# Calculate performance

accuracy = accuracy_score(y_test, model_rf.predict(X_test)) * 100

print(f"Random Forest Performance: {accuracy:.2f}%")

# --- 5. VISUALIZATION: THE "SMOKING GUNS" (Combined Labels) ---

importances = model_rf.feature_importances_

# Create the combined labels (Number + Name)

combined_labels = [sensor_labels.get(s, s) for s in sensors]

feat_importances = pd.Series(importances, index=combined_labels)

# Filter: Only keep sensors with significance > 0 (removes flat-line sensors)

# This ensures a clean, impactful chart for the presentation

non_zero_features = feat_importances[feat_importances > 0.005].sort_values(ascending=True)

plt.figure(figsize=(12, 10))

non_zero_features.plot(kind='barh', color='teal', edgecolor='black')

plt.title('Critical Indicators of Engine Failure\n(Sensor ID + Mechanical Description)', fontsize=16, fontweight='bold')

plt.xlabel('Significance Score (AI Model Importance)', fontsize=12)

plt.ylabel('Monitored Engine Component', fontsize=12)

# Adding a grid for readability

plt.grid(axis='x', linestyle='--', alpha=0.6)

plt.tight_layout()

plt.savefig('final_engine_graph.png')

print("\nGraph saved with IDs and Names as 'final_engine_graph.png'.")

plt.show()